|  | REQUEST FOR PROPOSALS |
| --- | --- |
|  | Jucicial Council of California addendum #1 Regarding: Energy efficiency Lighting Goods and Services admin-2018-08-lb PROPOSALS DUE: Wednesday, April 25, 2018 no later than 2:00 p.m. Pacific time |

This Addendum #1 hereby modifies the RFP as follows:

- Deletions in the RFP are shown in strikethrough font (strikethrough font); and insertions in the RFP are shown in bold underlined font (bold underlined font).  Paragraph numbers refer to the numbers in the original RFP.

- The following changes are made to the RFP:

2.0	DESCRIPTION OF GOODS AND SERVICES

The following changes are made to Section “G”, Item #7:

- ITEM #7 LED Lighting “Dimmable Linear T-8 replacement, 2 foot”

The Judicial Council anticipates order volumes could meet or exceed 50 pieces per order:

A 2 foot 8-10 Watt DIMMABLE LED Tube for replacement of T-8 Fluorescent. Provide dimmable LED lamp (minimum 2 lighting levels) for use with standard wall Toggle/Rocker switch.  Solution can be lamp & driver combination, plug & play and/or direct wire, or any combination to make a proposed solution for re-lamping in either plug & play, direct wire, or lamp plus driver modes.  Solution should meet or exceed as many of the Mandatory specifications below as possible:

Mandatory Specifications

- 15-18 8-10 Watt LED Tube for replacement of T-8 Fluorescent

- Beam Angle: between 100º - 140º

- L70 Lumen Maintenance (Hours) ≥50,000

- Power Factor ≥0.9

- Input Voltage AC 100-277v

- Frequency 50/60Hz

- Operating Temperature ≥ -20ºC ~ +40ºC

- IP Rating IP20

- Certification(s) UL, DLC, CUL, RoHS

- Color Temperature 4000-4200k

- Poly Carbonate lens, Glass prohibited

- Diffuser Frosted

- Efficiency ≥140 115 lm/w

- Lumens ≥2100 1300 lm

- CRI ≥80

4.0	RFP ATTACHMENTS

The following Attachment in Section 4.0 have been updated:

- Attachment 11 – Non-Cost Information XLS – revised to identify non-mandatory and optional specifications.